File SFPHYS1.DOC contains the following bookmarks. Select the topic you are trying to find from the list and double click the highlighted text.

Physics discussions in other files

| HREF="SFPHYS2.DOC | Properties of static magnetic and electric fields |
| --- | --- |
| HREF="SFPHYS3.DOC | Boundary conditions and symmetries |
| HREF="SFPHYS4.DOC | Numerical methods in Poisson and Pandira |
| HREF="SFPHYS5.DOC | RF cavity theory |

File SFPHYS1.DOC Table of Contents

XX. Theory of Electrostatics and Magnetostatics

A. Maxwell’s equations

B. Isotropic magnetostatics in Cartesian coordinates

C. Isotropic electrostatics in Cartesian coordinates

D. Isotropic magnetostatics in cylindrical coordinates

E. Isotropic electrostatics in cylindrical coordinates

F. Anisotropic magnetostatics in Cartesian coordinates

1. Easy axis in a fixed direction

2. Easy axis on an off-center circle

G. Anisotropic electrostatics in Cartesian coordinates

H. Anisotropic magnetostatics in cylindrical coordinates

I. Anisotropic electrostatics in cylindrical coordinates

# Theory of Electrostatics and Magnetostatics

This section is based upon the chapter of the same title in Part B, Chapter 13.1 by John L. Warren in the 1987 publication Reference Manual for the POISSON/SUPERFISH Group of Codes, LA-UR-87-126. We reproduce it here in essentially its original form as reference material to the physics in the Poisson Superfish codes. The equation sequence numbers are the same as in the original. We have corrected a few typographical errors in the original, including the following:

- All occurrences of ÑV in Equations XX-113 and XX-114 should be -ÑV.

- The DcY and DcX terms in Equation XX-115 should be negative.

- The correspondences in Equations XX-122 and XX-123 should have subscripts x and y interchanged on the left hand sides.

We have made a minor change in usage of the coercive force Hc. In the original version of Figure XX-5, which shows the B-H relation for an anisotropic material, the point on the horizontal axis was labeled “-Hc” implying that Hc itself is a positive number. To be consistent with the input of the quantities BCEPT and HCEPT in Automesh, we omit the minus sign in Figure XX-5. Automesh expects the sign of HCEPT to be positive.

## Maxwell’s equations

We begin with Maxwell’s Equations:

Ñ´E+¶B/¶t=0,	(XX-1)

Ñ´H−¶D/¶t=J,	(XX-2)

Ñ⋅B=0,	(XX-3)

Ñ⋅D=ρ.	(XX-4)

Equations XX-2 and XX=4 imply the equation of continuity

Ñ⋅J+¶ρ/¶t=0.	(XX-5)

Maxwell’s equations cannot be solved without assuming some relations among the vectors E, B, D, H, and J. In vacuum the relations are

D=e0E,	(XX-6)

H=B/m0,	(XX-7)

J=i=1NJi,	(XX-8)

ρ=i=1Nρi,	(XX-9)

where	dJi/dt=j¹ie/mjρjE+Jj´B,i=1,K,N,	(XX-10)

and where the summation is over all ionic species present. Equation XX-10 is the Lorentz force equation under the assumption that charge and mass are quantized and that relativistic and radiation damping effects can be ignored. This relation for J is just illustrative of the type of equation needed to close the system but will not be used here.

Both Poisson and Pandira can solve problems involving isotropic solid material. Pandira also can handle anisotropic materials, which will be discussed later. In isotropic solids, which are the only materials Poisson can handle, one can write with adequate generality

D=ex,t,EE=kex,t,Ee0E,	(XX-11)

H=gx,t,BB/m0,	(XX-12)

J=sx,t,EE,	(XX-13)

where the relative permittivity ke, the reluctivity g, and the conductivity s are usually piecewise constant functions of x. Note that the reluctivity g is the reciprocal of the relative permeability km. For fields slowly varying in time, the time dependence of e and g can be ignored. The case of rapidly oscillating fields is discussed in the section on HREF="sfphys5.doc#RFCavityTheory for the code Superfish. Let us look at static problems first.

Poisson and Pandira solve a generalized integral form of Poisson’s equation in two-dimensional, Cartesian coordinates or in cylindrically symmetric three-dimensional coordinates. This integral form of Poisson’s equation works for both magnetostatics and electrostatics. It also handles both isotropic and anisotropic materials. In the following subsections we will start with the integral equation and show how it is related to Maxwell’s and Poisson’s equations.

## Isotropic magnetostatics in Cartesian coordinates

The integral form of Equation XX-2 is

Cgx,BÑ´A⋅dl=m0VJ⋅da,	(XX-14)

where we have assumed that the displacement field D has no time dependence. The contour C encloses the area A as shown in Figure XX-1. The magnetic induction B is related to the vector potential A by the equation

B=Ñ´A.	(XX-15)

One obtains the generalized Poisson equation by the following two steps:

CgÑ´A⋅dl=AÑ´gÑ´A⋅da=Am0J⋅da,	(XX-16)

Ñ´gÑ´A=m0J.	(XX-17)

Figure XX-1. The geometry of the line and area integrals.

Equation XX-17 does not look like Poisson’s equation, but in two-dimensional Cartesian coordinates, it reduces to Poisson’s equation in the following way. Let us assume that A is only a function of two coordinates (x, y) and that B has only two nonzero components, Bx and By. Equation XX-15 requires that

Bx=¶Az¶y−¶Ay¶z=¶Az¶y,	(XX-18)

By=¶Ax¶z−¶Az¶x=-¶Az¶x,	(XX-19)

Bz=¶Ay¶x−¶Az¶y=0.	(XX-20)

Equation XX-20 is satisfied if Ax and Ay are required to be identically zero. This requirement also satisfies the gauge relation

Ñ⋅A=¶Ax¶x+¶Ay¶y+¶Az¶z=0.	(XX-21)

It is easily seen that Equation XX-17 becomes

¶¶xg¶Az¶x+¶¶yg¶Az¶y=-m0Jz,	(XX-22)

Jx=Jy=0.	(XX-23)

The latter equation forces the current to be parallel to A and Equation XX-22 is the generalized form of Poisson’s equation. Equation XX-14 in two dimensions reduces to

Cgx,y,B¶Az¶ydx−¶Az¶xdy=m0AJzdxdy.	(XX-24)

For a small enough area A, we can assume that g and Jz are nearly constant and that Az is a linear function of x and y, which we can write as

Az@-byx+bxy+a0.	(XX-25)

The integrals become

gbx,byCdxbx+gbx,byCdyby=m0JzA.	(XX-26)

This is a nonlinear equation with two unknowns, bx and by. It is the sort of equation that is created at each mesh point in the Poisson solver code. HREF="SFphys2.doc#FieldsAndDerivatives explains how the set of mesh-point equations are solved to obtain numerically the vector potential Az.

## Isotropic electrostatics in Cartesian coordinates

The integral form of Equation XX-4 is

Skex,EÑVx⋅ds=-1e0Vρxdv,	(XX-27)

where V(x) is the scalar potential and the integral on the right is over a volume V whose surface is denoted by S. The electric field is E given by

E=-ÑV.	(XX-28)

It is well known that Equation XX-27 is equivalent to the differential equation

Ñ⋅keÑV=-ρ/e0.	(XX-29)

In two-dimensional Cartesian coordinates this equation becomes

¶¶xkeV+¶¶ykeV=-ρ/e0.	(XX-30)

The similarity to Equation XX-22 is obvious. The integral form of the equation can also be made to resemble Equation XX-24 as follows. Suppose that ke, V(x), and r(x) are functions only of (x, y). Take the volume V to be a generalized cylinder of cross section A and length L as shown in Figure XX-2. Since V(x), is only a function of (x, y), ÑV has only x and y components. This statement says that the surface integral over the flat ends of the volume cannot contribute to the integral because the direction of the normal vectors for these areas is perpendicular to ÑV. The integral over the area of the cylindrical surface can be written as

Figure XX-2. Cylindrical volume in Cartesian coordinates.

The volume V has a uniform cross section of area A and length L. The inset shows the relation between the normal vector angle n and element of length dl.

SkeÑV⋅ds=CkeÑV⋅ndzdl,	(XX-31)

where n is the outward normal vector to the surface and dl is an element of length on the contour C. The integral over z evaluates to L and the quantity ndl can be written in Cartesian coordinates as

ndl=dlcosqex+dlsinqey.	(XX-32)

From Figure XX-2 we see that

dx=-dlsinq,	(XX-33)

dy=dlcosq,	(XX-34)

and hence	ndl=dy {ex−dx {ey.	(XX-35)

When this relation is put into Equation XX-31, the result is

SkeÑV⋅ds=-LCke¶V¶ydx−¶V¶xdy.	(XX-36)

The integral over the volume V can be written as

1e0VρdV=Le0Aρdxdy.	(XX-37)

This means that Equation XX-27 can be written as

Ckex,y,E¶V¶ydx−¶V¶xdy=1e0Aρdxdy.	(XX-38)

Comparison with Equation XX-24 makes it clear that there is a correspondence between the following pairs of quantities

V→Az,	(XX-39)

ke→g,	(XX-40)

1e0→m0,	(XX-41)

ρ→Jz,	(XX-42)

The same correspondences come out of a comparison between Equations XX-22 and XX-30.

## Isotropic magnetostatics in cylindrical coordinates

We assume that A is only a function of (r, z) and not the cylindrical angle q. The expressions for the components of B are

Br=Ñ´Ar=1r¶Az¶q−¶Aq¶z=-¶Aq¶z,	(XX-43)

Bz=1r¶¶rrAq−1r¶Ar¶q=1r¶¶rrAq,	(XX-44)

Bq=¶Ar¶z−¶Az¶r=0.	(XX-45)

Here we have assumed that B lies in the r-z plane. We choose the condition that

Ar=Az=0,	(XX-46)

which automatically satisfies the gauge condition Ñ⋅A=0. Equation XX-14 written in cylindrical coordinates is

Cgr¶¶zrAqdr−¶¶rrAqdz=m0AJqdrdz,	(XX-47)

Figure XX-3. Area and contour path in cylindrical coordinates.

If the contour is taken in the counterclockwise direction, then the area has a normal vector in the -eq direction. This fact is important in deriving Equation XX-47.

where we have introduced a factor of r into the derivative with respect to z for symmetry. Figure XX-3 shows the geometry. There is a correspondence used in the codes between quantities in cylindrical and Cartesian coordinates given by the relations

r→x,	(XX-48)

z→y,	(XX-49)

rAq→−Az,	(XX-50)

Jq→−Jz,	(XX-51)

gr→g.	(XX-52)

## Isotropic electrostatics in cylindrical coordinates

We assume that the scalar potential is only a function of (r, z). The electric field has only r and z components, which are given by

Er=-¶V¶r,	(XX-53)

Ez=-¶V¶z.	(XX-54)

The volume of integration in Equation XX-27 is a torus of cross sectional area A as illustrated in Figure XX-4. Integration over the angle q evaluates to 2p and Equation XX-27 can be expressed as

2pCkeÑV⋅nrdl=-2pe0Aρrdrdz.	(XX-55)

Figure XX-4. Volume in cylindrical coordinates.

The figure shows the geometry of the volume V, surface S, cross sectional area A, and contour C. The normal vector n to the area A is in the -eq direction.

By analogy to Equation XX-35, one can show that

ndl=dz {er−dr {ez,	(XX-56)

and hence Equation XX-55 becomes

Cker¶V¶zdr−¶V¶rdz=1e0Aρrdrdz.	(XX-57)

Comparison with the magnetostatic equation in Cartesian coordinates, Equation XX-24, gives the following correspondences

r→x,	(XX-58)

z→y,	(XX-59)

V→Az,	(XX-60)

ker→g,	(XX-61)

1e0→m0.	(XX-62)

rρ→Jz.	(XX-63)

Comparison with Equations XX-48 through XX-52 shows that Aq (r, z) is not analogous to V(r, z) in cylindrical coordinates.

## Anisotropic magnetostatics in Cartesian coordinates

Historically, there was an attempt to modify Poisson to handle two-dimensional anisotropic materials, but the code’s convergence was extremely poor. Halbach and Holsinger decided to write a new program called Pandira, which uses the so-called direct method rather than the successive over-relaxation method for the numerical solution of Maxwell’s equations. Maxwell’s equations become

Ñ´H=J,	(XX-64)

Ñ⋅B=0,	(XX-65)

B=Ñ´A,	(XX-66)

H=g⋅B/m0+Hc,	(XX-67)

where gis the reluctivity tensor and Hc is the magnetic field when B = 0, that is, the permanent magnetic field. Pandira handles anisotropic materials having an “easy axis” and a “hard axis” perpendicular to the easy axis,

H=H||+H,	(XX-68)

H||=g||B||/m0+Hc,	(XX-69)

H=gB/m0,	(XX-70)

or one can write the inverse relations

B||=km||m0H||+Br,	(XX-71)

B=kmm0H,	(XX-72)

where	g||=1/km||,	(XX-73)

g=1/km,	(XX-74)

Br=-km||m0Hc,	(XX-75)

and where B|| is parallel to the easy axis and B^ is along the hard axis. The permeability relations are different in the two special directions. Figure XX-5 shows typical relations. The easy axis is characterized by the coercive force Hc and the remanent field Br. Program Pandira allows two geometries for the easy axis. In the first geometry the easy axis is independent of position in the material. In the second geometry, the direction of the easy axis changes along the circumference of a circle that is not necessarily concentric with the origin of coordinates. These two geometries are explained below.

Figure XX-5. B-H relationships for an anisotropic material.

The left figure shows the coercive force Hc (in Oersteds) and the remanent field Br (in Gauss) for fields parallel to the easy axis. For a non-permanent (but anisotropic) magnetic material, the curve goes through the origin like it does for the direction perpendicular to the easy axis shown at right.

Figure XX-6. Unit vectors for anisotropic materials.

This figure defines the angle jE and the two unit vectors e||and ethat lie along the easy and hard axes.

### Easy axis in a fixed direction

Figure XX-6 shows the directions of the easy and hard axes relative to the coordinate axes in the region of interest. The field B can be expressed in terms of the unit vectors for the two coordinate systems as

B=BXeX+BYeY=B||e||+Be.	(XX-76)

The unit vectors are related by the matrix transformation

.	(XX-77)

The field H takes the form

H=g||B||/m0+Hce||+,	(XX-78)

from which one can deduce that

H=.	(XX-79)

By using the inverse transformation on the unit vectors, namely,

eXeY=cosjE−sinjEsinjEcosjE,	(XX-80)

one can show that

B=BXcosjE+BYsinjEe||+−BXsinjE+BYcosjEe.	(XX-81)

This gives B|| and B^ (and hence H|| and H^) in terms of BX and BY. After making substitutions and solving for HX and HY one obtains

HX=,	(XX-82)

HY=sin2jE.	(XX-83)

These equations can be written in matrix form as

HXHY=1m0gXXgXYgXYgYYBxBy−HcXHcY,	(XX-84)

Figure XX-7. Permanent-magnet quadrupole.

This example, which is in the HREF="sfexmpl2.doc#PermanentMagnetQuadWith16Wedges directory, is a magnet constructed from wedge-shaped blocks of permanent magnet material with the easy axis in each block having a different orientation.

Figure XX-8. Definitions of parameters in off-center anisotropic materials.

where the following equations define the symmetric reluctivity tensor g and the components of the coercive force Hc:

gXX=g||cos2jE+gsin2jE,	(XX-85)

gXY=12sin2jE,	(XX-86)

gYY=g||sin2jE+gcos2jE,	(XX-87)

HcX=HccosjE,	(XX-88)

HcY=HcsinjE.	(XX-89)

### Easy axis on an off-center circle

Pandira includes an option that allows the easy axis direction to be a function of angle around the center of a circle. Although there is no natural material for which the direction of the easy axis is a function of position in the plane, one can construct a magnet from blocks of permanent magnet material with each block having its own orientation as shown in Figure XX-7. One can think of the wedge magnet as approximating the idealized material. The chapter on HREF="sfcodes.doc#VariableEasyAxisDirection describes how to use this option in an input file. The directory HREF="sfexmpl2.doc#PMQUADdirectory includes a sample input file for a 16-wedge quadrupole magnet like the one in Figure XX-7. For comparison, this directory also has an example of the idealized case of continuously varying easy axis direction. We believe that variable-easy-axis option was added to Pandira because early versions of the code did not allow enough material types to solve the real problem. We recommend setting up the actual geometry for design work. The idealized option in Pandira may still be useful for visualizing a concept.

The center of the circle need not coincide with the origin of coordinates. Figure XX-8 shows the geometry of the easy axis in such a material. The origin of the easy axis is determined by the three parameters XA, YA, and jA. Relating these parameters to reluctivity tensor g and the coercive force Hc is a matter of coordinate transformations. There are three coordinate systems that enter into the formulation,

.	(XX-90)

In the (ex,eh) system, the tensor g and the vector Hc are the same as described in Equations XX-85 through XX-89 above. The axes (ex,eh)change as a function of (x, y). The (x, y) coordinates are related to the (X,Y) coordinates by a linear transformation. Let

X=XeX+YeY=XA+xeX+YA+yeY,	(XX-91)

and	exeh=cosqsinq−sinqcosqeXeY,	(XX-92)

where	cosq=X−XAX−XA2+Y−YA2,	(XX-93)

and	sinq=Y−YAX−XA2+Y−YA2.	(XX-94)

From Figure XX-8 one sees that

.	(XX-95)

Either by multiplication of the matrices or by looking at Figure XX-8 again, one can show that

.	(XX-96)

The derivation of the components of g can be completed as we did before for the fixed direction case. The only difference is that jE is replaced by jA + q(X, Y). The reluctivity tensor g and the coercive force Hc are now functions of the position in the material. In an MT namelist in the Automesh input file, the parameters are defined as follows:

AEASY=jE,GAMPER=g,HCEPT=Hc,	(XX-97)

and g|| is determined from Br, that is, from Figure XX-5, where we see that

g||=Br/m0Hc.	(XX-98)

With this information as a background, we now write down the integral equation used in Pandira to find the vector potential, namely,

Cg⋅Ñ´A+m0Hc⋅dl=m0AJ⋅da.	(XX-99)

In Cartesian coordinates we once more choose the gauge

Ñ⋅A=0,	(XX-100)

which requires that

A=Azez,	(XX-101)

and	Jx=Jy=0.	(XX-102)

Equation XX-24 becomes

CgXX¶Az¶y−gXY¶Az¶x+m0HcXdx
+gXY¶Az¶y−gYY¶Az¶x+m0HcYdy=m0AJzdxdy.	(XX-103)

This complication does not prevent one from applying the same numerical method discussed in HREF="SFphys2.doc#FieldsAndDerivatives to find the solution. If Az is a linear function of x and y as in Equation XX-25, then Equation XX-103 reduces to

gXXCdx+gXYCdybx+gXYCdx+gYYCdyby=
m0JzA−HcXCdx−HcYCdy.	(XX-104)

This equation, like Equation XX-26, is still nonlinear in bx and by. Since the g’s are functions of bx and by, the equation must be solved iteratively.

## Anisotropic electrostatics in Cartesian coordinates

The basic equations for dielectric materials are

Ñ⋅D=ρ.	(XX-105)

Ñ´E=0,	(XX-106)

E=-ÑV,	(XX-107)

D=e0ke⋅E+Dc,	(XX-108)

As in the magnetostatic case, one can define an easy axis and a hard axis for the material. This definition results in relations analogous to Equations XX-68 though XX-89 with the correspondences

H→D,	(XX-109)

B→E,	(XX-110)

g→ke,	(XX-111)

m0→1/e0.	(XX-112)

The coordinate transformations expressed in Equations XX-90 though XX-96 are the same. The integral equation for the scalar potential is a generalization of Equation XX-27, which was derived with the assumptions that ke = 1 and Dc = 0. Divide equation XX-108 by e0 and substitute for E from equation XX-107 to see that ke(-ÑV) can be replaced by the quantity ke(-ÑV) + Dc/e0, resulting in the equation

Ske⋅-ÑV+Dc/e0⋅ds=1e0Vρdv.	(XX-113)

The derivation leading to Equation XX-36 is essentially unchanged. The final result is

Ske⋅-ÑV+Dc/e0⋅ds=

−LCke⋅-ÑV+Dc/e0ydx−Cke⋅-ÑV+Dc/e0xdy.	(XX-114)

Again, the integral over z in the volume integral on the right-hand side of Equation XX-113 evaluates to L, so Equation XX-113 can be written in the form

CkeYX¶V¶x+keYY¶V¶y−DcYe0dx
−CkeXX¶V¶x+keXY¶V¶y−DcXe0dy=1e0Aρdxdy.	(XX-115)

A comparison with the magnetostatic result, Equation XX-103, reveals distinct differences in interpretation. The two equations take the same form if we make the substitutions

V→Az,	(XX-116)

ρ→Jz,	(XX-117)

e0→1m0,	(XX-118)

keXX→gYY,	(XX-119)

keXY=keYX→−gXY,	(XX-120)

keYY→gXX,	(XX-121)

DcY→−HcX,	(XX-122)

DcX→HcY.	(XX-123)

The interchange of components and signs is equivalent to changing jE to jE + 90 degrees when the easy axis has a fixed direction. This is not simply an interchange of hard and easy axes.

## Anisotropic magnetostatics in cylindrical coordinates

To find the correspondence between the cylindrical and Cartesian cases, one must return to Equation XX-99 and express the Ñ operator and cross products in cylindrical coordinates. By following the steps taken in deriving Equation XX-47 one finds that

CgRRr¶¶z−rAq−gRZr¶¶r−rAq+m0HcRdr
+CgRZr¶¶z−rAq−gZZr¶¶r−rAq+m0HcZdz
=m0A−Jqdrdz.	(XX-124)

Comparison with Equation XX-103 gives the correspondences

rAq→−Az,	(XX-125)

Jq→−Jz,	(XX-126)

gRRr→gXX,	(XX-127)

gRZr→gXY,	(XX-128)

gZZr→gYY,	(XX-129)

HcR→HcX,	(XX-130)

HcZ→HcY,	(XX-131)

which are the analogue of Equations XX-50 through XX-52.

## Anisotropic electrostatics in cylindrical coordinates

The starting point is Equation XX-113, which must be expressed cylindrical coordinates. The resulting equation is

C−rkeZR¶V¶r−rkeZZ¶V¶z+1e0−rDcZdr
+CrkeRR¶V¶r+rkeRZ¶V¶z+1e0rDcRdz
=1e0Arρdrdz.	(XX-132)

Comparison with Equation XX-103 gives the correspondences

V→Az,	(XX-133)

rρ→Jz,	(XX-134)

e0→1m0,	(XX-135)

rkeRR→−gYY,	(XX-136)

rkeRZ=rkeZR→gXY,	(XX-137)

rkeZZ→−gXX,	(XX-138)

rDcR→HcY,	(XX-139)

rDcZ→HcX.	(XX-140)